SH21789A  Abel closed cup Flash point tester

SH21789A Abel closed Flash point tester suitable for the determination of petroleum products and other liquids with flash point between -30 °C ~ 70 °C, precision is only suitable for samples with flash point between -5 °C ~ 66.5 °C, not suitable for water-based coatings. Can be used for flash point determination of aviation (jet) fuels.

SH21789A Abel closed Flash point tester meets the requirements of GB/T 21789, ISO 13736 Test method for determination of flash point of petroleum products and other liquids (Abel closed cup method).

| Model | SH21789A |
| --- | --- |
|  | Flash point tester (Abel closed cup method) |
| Manufacturer | Shandong Shengtai Instrument Co., Ltd |
| Test item | It is suitable for determining petroleum products and other liquids with flash point between -30 °C ~ 70 °C, and the precision is only suitable for samples with flash point between -5 °C ~ 66.5 °C, not suitable for water-based coatings |
| Meet the standard | GB/T 21789、ISO 13736 Determination of flash point of petroleum products and other liquids (Abel closed cup method) |
| technical characteristics or parameters | Measuring range：-30～70℃ |

Features

1. All stainless steel gas pipe

2. High-precision needle valve

3. Standard reagent calibration

4. Optional  low temperature circulation bath

Technical parameters

| Power supply | AC 220V± 10% 50Hz |
| --- | --- |
| Power | 350W |
| Ignition method | gas ignition |
| Measuring range | Room temperature to 70℃, or -30 to 70℃ (Optional low-temperature circulating bath) |
| Stirring speed | 30 r/min |
| Repetition error | <±1°C |
| Environmental requirements | temperature 5~40°C; Humidity≤85% |
| Packaging abel flash point | size: 560*550*650mm Weight: 45kg |
| Abel flash point | size: 480*450*550mm Weight: 40kg |

Packaging and  delivery

| No. | Name | Unit | Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | host | Set | 1 |  |
| 2 | Fuse wire | pieces | 1 |  |
| 3 | Test cup | only | 1 |  |
| 4 | Test cup lid | Set of | 1 |  |
| 5 | Thermometer | Piece | 2 |  |
| 6 | Thermometer sleeve | pieces | 2 |  |
| 7 | instructions | copy | 1 |  |
| 8 | Packing list | copy | 1 |  |
| 9 | Warranty card | copy | 1 |  |