SH0702   Residual fuel oil total sediment tester

The residual fuel oil total sediment tester is designed and manufactured in accordance with SH/T0701 and SH/T0702 Residual fuel oil total sediment determination method (aging method, thermal filtration method). This method is applicable to the determination of total sediment in residual fuel oil with viscosity less than 55mm2/s at 100 ℃ and distillate fuel oil mixed with residual components.

This instrument is applicable to the total sediment in residual fuel oil and distillate fuel oil containing residual components.

Product features

- Equipped with a special aging bath;

- Standard filter elements with heating and cooling coils;

- Glass fiber filter paper aperture 0.0016mm diameter 47mm;

- Stainless steel ball valve switches heat source and cold source;

- Digital display time relay controls working time;

- Microcomputer PID temperature controller, PT100 temperature sensor, high temperature control accuracy;

- Air condenser specification： Made of glass, with an outer diameter of 8mm and a length of 400mm.

Technical parameter

| Applicable standard | SH/T0701 SH/T0702,ASTMD 4870 |
| --- | --- |
| Working power supply | AC220V 50HZ |
| Overall power | 1000W |
| Filter paper | aperture 0.0016mm diameter 47mm |
| Vacuum pressure | 40kpa |
| Test bit | 2 units |
| Test range | 0.5%(m/m) |
| Waste flask capacity | 500ml |
| Test sample | 25g |
| Instrument size：640*330*570mm Net weight：32KG |  |

Packing List

| S/N | Name | Quantity | Unit | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Host | 1 | Set |  |
| 2 | Suction bottle | 1 | Piece |  |
| 3 | Suction filter pump | 1 | Piece |  |
| 4 | Power cord | 1 | Piece |  |
| 5 | Instructions | 1 | copy |  |
| 6 | Packing list | 1 | copy |  |
| 7 | Certificate warranty card | 1 | copy |  |

Optional accessories:

1. Electronic balance (one ten-thousandth)

2. Drying oven

3. Glass dryer (with silica gel desiccant)

4. A graduated wash bottle or a 25ml syringe

5. Stirring rod glass and 30ml glass beaker

6. High-speed mixer, 400r/min

7. Weighing bottle or weighing dish

8. Glass fiber filter paper